WELDING PROCEDURE SUMMARY

Ridgeline Fabrication  ·  Rev. 04/2025

QUALIFIED PROCESSES

- GMAW (MIG) — primary process for carbon steel

- FCAW — heavy plate, 0.375 in and above

- GTAW (TIG) — stainless and aluminum, and anything cosmetic

We are not qualified for SMAW production work. Field repairs only, with the quality lead’s approval.

MATERIALS WE WELD

| Material | Process | Filler |
| --- | --- | --- |
| A36 carbon steel | GMAW / FCAW | ER70S-6 / E71T-1 |
| A572 Gr 50 | FCAW | E71T-1 |
| 304 stainless | GTAW / GMAW | ER308L |
| 316 stainless | GTAW | ER316L |
| 6061 aluminum | GTAW | ER4043 |
| 5052 aluminum | GTAW | ER5356 |

CERTIFICATION

- All production welders hold current AWS D1.1 qualification for the positions they run

- Aluminum work requires separate D1.2 qualification. Three welders currently hold it.

- Qualifications are re-verified every 6 months. Bernice tracks the expiration dates.

WHAT REQUIRES DOCUMENTATION

- Any structural weldment going into a permitted building

- Anything the customer’s print calls out as requiring a WPS or PQR

- Third-party inspected work. Add a week to the lead time and tell scheduling early.

WHAT WE DO NOT DO

- Pressure vessel work. No ASME stamp.

- Titanium or exotic alloys

- Structural aluminum in a permitted application